Metrix OX7000 DSO 1 GS/s (VISA)

________________________________________________________________________________

Introduction:

This instrument driver provides programming support for Metrix OX7000 DSO 1 GS/s (VISA).

It contains functions for opening, configuring, taking measurements from, and

closing the instrument.

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

For RS-232 instrument drivers:

-  the instrument is connected to the RS-232 interface.

-  the COM port, baud rate, parity, and timeout supplied to the

initialize function must match the settings of the instrument.

For ETERNET

________________________________________________________________________________

Error and Status Information:

Each function in this instrument driver returns a status code that

either indicates success or describes an error or warning condition.

Your program should examine the status code from each call to an

instrument driver function to determine if an error occurred.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

The description of each instrument driver function lists possible error

codes and their meanings

________________________________________________________________________________

How To Use This Document:

Use this document as a programming reference manual.

It describes each function in the

Metrix OX7000 DSO 1 GS/s (VISA)

instrument.  The functions appear in alphabetical order, with a description

of the function and its C syntax, a description of each parameter, and a list

of possible error codes.

________________________________________________________________________________

Function Tree Layout:

Class/Panel Name:                          Function Name:

Initialize                              mxOX7000_init

Application Functions

Example                              mxOX7000_example

Configure Functions

Vertical Setup                       mxOX7000_verticalSetup

Horizontal

Horizontal Setup                  mxOX7000_horizontalSetup

Horizontal Display                mxOX7000_horizontalDisplay

FFT                               mxOX7000_FFT

Trigger

General Trigger                   mxOX7000_setGeneralTrigger

Pulse Trigger                     mxOX7000_setPulsTrigger

Delayed Trigger                   mxOX7000_setDelayedTrigger

Count Trigger                     mxOX7000_setCountTrigger

Video Trigger                     mxOX7000_setVideoTrigger

Display

Options                           mxOX7000_setOptionDisplay

Waveform                          mxOX7000_setWaveformDisplay

Cursors                           mxOX7000_setCursors

Hardcopy Setup                       mxOX7000_setHardcopy

Action/Status Functions

Autoset                              mxOX7000_runAutoset

Run/Stop                             mxOX7000_runAcquisition

Hardcopy                             mxOX7000_runHardcopy

Data

Read Cursors                         mxOX7000_readCursors

Read Measurements                    mxOX7000_readMeasurement

Waveform

Read Channel To Array             mxOX7000_readChToArr

Read Channel To File              mxOX7000_readChToFile

Read File To Array                mxOX7000_readFileToArr

Write File To Memory              mxOX7000_writeFileToMem

Transfer

Save/Recall Setup To/From File    mxOX7000_saveSetup

File Transfer                     mxOX7000_fileTransfer

Utility Functions

Write To Instrument                  mxOX7000_writeInstrData

Read Instrument Data                 mxOX7000_readInstrData

Reset                                mxOX7000_reset

Self-Test                            mxOX7000_selfTest

Error-Query                          mxOX7000_errorQuery

Error Message                        mxOX7000_errorMessage

Revision Query                       mxOX7000_revisionQuery

METER

MeterConfig                          mxOX7000_Meterconfig

Read MeterMeas                       mxOX7000_readMeterMeas

Close                                   mxOX7000_close

________________________________________________________________________________

Metrix OX7000 DSO 1 GS/s (VISA)

This instrument module provides programming support for the Metrix OX7000

DSO 1GS/s (VISA).  The module is divided into the following functions:

Functions/Classes:

(1) Initialize:

This function initializes the instrument and sets it to a default

configuration.

(2) Application Functions: (Class)

This class contains high-level, test and measurement routines.  These

examples call other instrument driver functions to configure, start, and

read from the instrument.

(3) Configuration Functions: (Class)

This class of functions configures the instrument by setting acquisition

and system configuration parameters.

(4) Action/Status Functions: (Class)

This class of functions begins or terminates an acquisition. It also

provides functions which allow the user to determine the current status

of the instrument.

(5) Data Functions: (Class)

This class of functions transfers data to or from the instrument.

(6) Utility Functions: (Class)

This class of functions provides lower level functions to communicate

with the instrument, and change instrument parameters.

(7) Close:

This function takes the instrument offline.

________________________________________________________________________________

The following functions are in alphabetical order.

________________________________________________________________________________

mxOX7000_close

ViStatus mxOX7000_close (ViSession instrumentHandle);

Purpose

This function performs the following operations:

viClose (instrSession) and viClose (rmSession).

Notes:

(1) The instrument must be reinitialized to use it again.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_errorMessage

ViStatus mxOX7000_errorMessage (ViSession instrumentHandle,

ViStatus errorCode,

ViChar _VI_FAR errorMessage[]);

Purpose

This function takes the Status Code returned by the instrument driver

functions, interprets it and returns it as a user readable string.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  VI_NULL

errorCode

Variable Type       ViStatus

This control accepts the Status Code returned from the instrument

driver functions.

Default Value:

0 - VI_SUCCESS

errorMessage

Variable Type       ViChar[]

This control returns the interpreted Status Code as a user readable

message string.

Notes:

(1) The array must contain at least 256 elements ViChar[256].

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

3FFF0085  Unknown status.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_errorQuery

ViStatus mxOX7000_errorQuery (ViSession instrumentHandle,

ViPInt32 errorCode,

ViChar _VI_FAR errorMessage[]);

Purpose

This function reads an error code from the instrument's error queue.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

errorCode

Variable Type       ViInt32 (passed by reference)

This control returns the error code read from the instrument's error

queue.

errorMessage

Variable Type       ViChar[]

This control returns the error message string read from the

instrument's error message queue.

Notes:

(1) The array must contain at least 256 elements ViChar[256].

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_example

ViStatus mxOX7000_example (ViSession instrumentHandle, ViInt16 source,

ViInt16 time_base, ViInt16 voltageRange,

ViInt16 coupling,

ViReal64 _VI_FAR waveformArray[],

ViPInt32 number_of_samples, ViPReal64 xStart,

ViPReal64 xIncrement);

Purpose

This function is an example of this driver's functions use.

The instrument is set to the single shot mode.

User can change the source channel, the timebase, the input voltage range

and the input coupling.

A trigger is then sent to the instrument.

The function waits until the acquisition is completed.

Channel is read out from the instrument.

Waveform is interpolated if needed.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

source

Variable Type       ViInt16

This control selects the source channel.

Valid Values:

0 - CH1  (Default Value)

1 - CH2

2 - CH3

3 - CH4

time_base

Variable Type       ViInt16

This control selects the main timebase  value..

Valid Values:

0  - 1ns

1  - 2ns

2  - 5ns

3  - 10ns

4  - 25ns

5  - 50ns

6  - 125ns

7  - 250ns

8  - 500ns - (Default Value)

9  - 1s

10 - 2.5s

11 - 5s

12 - 10s

13 - 20s

14 - 50s

15 - 100s

16 - 200s

17 - 500s

18 - 1ms

19 - 2ms

20 - 5ms

21 - 10ms

22 - 20ms

23 - 50ms

24 - 100ms

25 - 200ms

26 - 500ms

27 - 1s

28 - 2s

29 - 5s

30 - 10s

31 - 20s

32 - 50s

33 - 100s

34 - 200s

voltageRange

Variable Type       ViInt16

This control sets vertical sense for the specified channel.

Valid Values:

0  - 2mV/div

1  - 5mV/div

2  - 10mV/div

3  - 20mV/div

4  - 50mV/div

5  - 100mV/div

6  - 200mV/div

7  - 500mV/div

8  - 1V/div

9  - 2V/div

10 - 5V/div

11 - 10V/div  (Default Value)

12 - 20V/div

13 - 50V/div

14 - 100V/div

15 - 200V/div

16 - 500V/div

17 - 1kV/div

Note : This control value must be consistent with the probe value.

See the Digiscope 2000 Operating Instructions manual for more

details. If it is inconsistent an error is generated :

VI_ERROR_INCONSISTENT_PARAMETER7 (0xBFFC0907)

coupling

Variable Type       ViInt16

This control selects the input coupling of the selected channel.

Valid Values:

0 - AC  (Default Value)

1 - DC

2 - GND

waveformArray

Variable Type       ViReal64[]

This control specifies the name of the array in which to place the

waveform data.

Notes:

(1) The Real Array must be large enough to hold the requested points.

number_of_samples

Variable Type       ViInt32 (passed by reference)

This control returns the number of valid points read from the

selected channel.

xStart

Variable Type       ViReal64 (passed by reference)

This control returns the time of the first data point in seconds

(with respect to the trigger point).

xIncrement

Variable Type       ViReal64 (passed by reference)

This control returns the incrementing value in seconds for the X

axis.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Channel) out of range.

BFFC0003  Parameter 3 (Timebase) out of range.

BFFC0004  Parameter 4 (Voltage Range) out of range.

BFFC0005  Parameter 5 (Coupling) out of range.

BFFC0803  Error scanning response.

BFFC0900  Instrument specific error.

BFFC090C  Channel is not active.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_FFT

ViStatus mxOX7000_FFT (ViSession instrumentHandle, ViBoolean FFT,

ViInt16 calculate, ViBoolean scale);

Purpose

This function configures the FFT mode.

The following settings are available :

- FFT ON or OFF

- Calculate

- Scale

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

FFT

Variable Type       ViBoolean

This control valid the FFT mode.

Valid Values:

VI_OFF (0) - OFF  (Default Value)

VI_ON  (1) - ON

calculate

Variable Type       ViInt16

This control selects the calculate mode.

Valid Values:

0 - Rectangular  (Default Value)

1 - Hamming

2 - Hanning

3 - Blackman

scale

Variable Type       ViBoolean

This control selects the Scale.

Valid Values:

VI_ON  (1) - Log scale   (Default Value)

VI_OFF (0) - Linear scale

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Trigger Source) out of range.

BFFC0003  Parameter 3 (Standard) out of range.

BFFC0004  Parameter 4 (Polarity) out of range.

BFFC0005  Parameter 5 (Field) out of range.

BFFC0006  Parameter 6 (line) out of range.

BFFC0803  Error scanning response

BFFC0900  Instrument specific error.

BFFC0906  Parameter 6 (Line) is inconsistent.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_fileTransfer

ViStatus mxOX7000_fileTransfer (ViSession instrumentHandle,

ViString fileName, ViInt16 sourceDrive,

ViBoolean overwriteFile);

Purpose

This function performs a file transfer.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

fileName

Variable Type       ViString

This function specifies the name of the data file.

Default Value: ""

Notes:

(1) When reading a file from Computer, precede all backslashes with

an additional backslash, e.g. if the file is in the \cvi\instr

directory, enter the filename as "\\cvi\\instr\\anyfile".

(2) File name must be 15-character long max + ".???" extension.

The extention must be in capital letter.

If not an error is generated :0xBFFC0002

(3) If the file already exists on the target drive and overwrite is

not selected an error is generated : 0xBFFC090E

(4) If file does not exist on the source drive an error is generated

: 0xBFFC090D

sourceDrive

Variable Type       ViInt16

This control selects the source drive.

Valid Values:

0 - Computer  (Default Value)

1 - MTX3x52 Memory

overwriteFile

Variable Type       ViBoolean

This function specifies whether or not to write over the specified

file if it already exists on disk.

Valid Range:

VI_OFF (0) - No  (Default Value)

VI_ON  (1) - Yes

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Filename) out of specifications.

BFFC0003  Parameter 3 (Source Drive) out of range.

BFFC0004  Parameter 4 (Overwrite File?) out of range.

BFFC0005  Parameter 5 (Target Drive) out of range.

BFFC0800  Error opening file.

BFFC0801  Error writing to file.

BFFC0803  Error scanning response

BFFC0900  Instrument specific error.

BFFC090D  File not found.

BFFC090E  File already exists.

BFFC0910  Invalid file content.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_horizontalDisplay

ViStatus mxOX7000_horizontalDisplay (ViSession instrumentHandle,

ViInt16 trace, ViInt16 x,

ViInt16 y);

Purpose

This function configures the horizontal display of the instrument.

The following settings are available :

- Trace

- X (for XY mode)

- Y (for XY mode)

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

trace

Variable Type       ViInt16

This control selects the traces to be displayed.

Valid Values:

0 - Oscilloscope  (Default Value)

1 - XY

x

Variable Type       ViInt16

This control selects the X input for XY mode.

Valid Values:

1 - 1  (Default Value)

2 - 2

3 - 3

4 - 4

y

Variable Type       ViInt16

This control selects the Y input for XY mode.

Valid Values:

1 - 1  (Default Value)

2 - 2

3 - 3

4 - 4

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Trace) out of range.

BFFC0003  Parameter 3 (X) out of range.

BFFC0004  Parameter 4 (Y) out of range.

BFFC0005  Parameter 5 (Roll) out of range.

BFFC0006  Parameter 6 (Cumul) out of range.

BFFC0803  Error scanning response.

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_horizontalSetup

ViStatus mxOX7000_horizontalSetup (ViSession instrumentHandle,

ViInt16 range,

ViReal64 pretigg_value);

Purpose

This function configures the horizontal settings of the instrument.

The following settings are available :

- Pretrigger

- Range (the main timebase)

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

range

Variable Type       ViInt16

This control selects the main timebase  value.

Valid Values:

0  - 1ns

1  - 2ns

2  - 5ns

3  - 10ns

4  - 25ns

5  - 50ns

6  - 125ns

7  - 250ns

8  - 500ns - (Default Value)

9  - 1s

10 - 2.5s

11 - 5s

12 - 10s

13 - 20s

14 - 50s

15 - 100s

16 - 200s

17 - 500s

18 - 1ms

19 - 2ms

20 - 5ms

21 - 10ms

22 - 20ms

23 - 50ms

24 - 100ms

25 - 200ms

26 - 500ms

27 - 1s

28 - 2s

29 - 5s

30 - 10s

31 - 20s

32 - 50s

33 - 100s

34 - 200s

pretigg_value

Variable Type       ViReal64

This control selects the pretrig value ().

Default Value: 0.0

Valid Values are between -10 * Range and 25 * Range.

Range     Max Values    Min Values      Middle Values

200s   ->  5000        -2000            -1000

100s   ->  2500        -1000            -500

50s    ->  1250        -500             -250

20s    ->  500         -200             -100

10s    ->  250         -100             -50

5s     ->  125         -50              -25

2s     ->  50          -20              -10

1s     ->  25          -10              -5

500ms  ->  12500       -5000            -2500

200ms  ->  5000        -2000            -1000

100ms  ->  2500        -1000            -500

50ms   ->  1250        -500             -250

20ms   ->  500         -200             -100

10ms   ->  250         -100             -50

5ms    ->  125         -50              -25

2ms    ->  50          -20              -10

1ms    ->  25          -10              -5

500us  ->  12500       -5000            -2500

200us  ->  5000        -2000            -1000

100us  ->  2500        -1000            -500

50us   ->  1250        -500             -250

20us   ->  500         -200             -100

10us   ->  250         -100             -50

5us    ->  125         -50              -25

2us    ->  50          -20              -10

1us    ->  25          -10              -5

500ns  ->  12500       -5000            -2500

200ns  ->  5000        -1000            -1000

100ns  ->  2500        -1000            -500

50ns   ->  2500        -1000            -500

20ns   ->  2500        -1000            -500

10ns   ->  2500        -1000            -500

5ns    ->  2500        -1000            -500

2ns    ->  2500        -1000            -500

1ns    ->  2500        -1000            -500

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Size) out of range.

BFFC0003  Parameter 3 (Pretrig) out of range.

BFFC0004  Parameter 4 (Delay) out of range.

BFFC0005  Parameter 5 (RangeA) out of range.

BFFC0006  Parameter 6 (DelayA) out of range.

BFFC0007  Parameter 7 (RangeB) out of range.

BFFC0008  Parameter 8 (DelayB) out of range.

BFFC0803  Error scanning response.

BFFC0900  Instrument specific error.

BFFC0907  Parameter 7 (RangeB) is inconsistent.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_init

ViStatus mxOX7000_init (ViRsrc resourceName, ViBoolean IDQuery,

ViPSession instrumentHandle);

Purpose

This function performs the following initialization actions:

- Opens a session to the Default Resource Manager resource and a session

to the specified device using the interface and address specified in the

Resource_Name control.

- Performs an identification query on the Instrument.

- Serial Parameters are: Speed = 115200, Parity = None, Data = 8 bits,

Protocol = RTS/CTS (hard)

- Sends initialization commands to the instrument that set any necessary

programmatic variables such as Headers Off, Short Command form, and Data

Transfer Binary to the state necessary for the operation of the

instrument driver.

- Returns an Instrument Handle which is used to differentiate between

different sessions of this instrument driver.

- Each time this function is invoked a Unique Session is opened.  It is

possible to have more than one session open for the same resource.

Parameter List

resourceName

Variable Type       ViRsrc

This control specifies the interface and address of the device that

is to be initialized (Instrument Descriptor). The exact grammar to be

used in this control is shown in the note below.

Default Value:  "ASRL1"

Notes:

(1) Based on the Instrument Descriptor, this operation establishes a

communication session with a device.  The grammar for the Instrument

Descriptor is shown below.  Optional parameters are shown in square

brackets ([]).

Interface   Grammar

------------------------------------------------------

Serial      ASRL<port>[::INSTR]

TCPIP       TCPIP[board]::host address::23::SOCKET

The ASRL keyword is used with Serial instruments.

The default value for optional parameters are shown below.

Optional Parameter          Default Value

-----------------------------------------

port                        1 - 16

board                       0

host address                IP address

IDQuery

Variable Type       ViBoolean

This control specifies if an ID Query is sent to the instrument

during the initialization procedure.

Valid Range:

VI_OFF (0) - Skip Query

VI_ON  (1) - Do Query (Default Value)

Notes:

(1) Under normal circumstances the ID Query ensures that the

instrument initialized is the type supported by this driver. However

circumstances may arise where it is undesirable to send an ID Query

to the instrument.  In those cases; set this control to "Skip Query"

and this function will initialize the selected interface, without

doing an ID Query.

instrumentHandle

Variable Type       ViSession (passed by reference)

This control returns an Instrument Handle that is used in all

subsequent function calls to differentiate between different sessions

of this instrument driver.

Notes:

(1) Each time this function is invoked a Unique Session is opened.

It is possible to have more than one session open for the same

resource.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (ID Query) out of range.

BFFC0003  Parameter 3 (Reset Device) out of range.

BFFC0011  Instrument returned invalid response to ID Query

BFFC0803  Error Scanning Response

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_Meterconfig

ViStatus mxOX7000_Meterconfig (ViSession instrumentHandle,

ViInt16 input, ViInt16 measure,

ViInt16 coupling, ViBoolean autoRange,

ViInt16 voltRang, ViInt16 ohmRang,

ViInt16 capaRang);

Purpose

This function configures the meter mode of the instrument.

The following settings are available :

- Input

- Measure

- Coupling

- AutoRange

- VoltRang

- OhmRang

- CapaRang

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

input

Variable Type       ViInt16

This control selects the channel to configure.

Valid Values:

1 - CH1  (Default Value)

2 - CH2

3 - CH3

4 - CH4

measure

Variable Type       ViInt16

This command selects the measurement function on channel 1.

Valid Values:

0  - Voltage

1  - Resistance

2  - Continuity

3  - Capacitor

4  - Diode

5  - PT100

coupling

Variable Type       ViInt16

This command affects the coupling of the selected channel.

Valid Values:

0 - AC

1 - DC  (Default Value)

2 - GND

autoRange

Variable Type       ViBoolean

This command authorizes or prohibits the autoranging of the selected

channel.

Valid Values:

VI_ON  (1) - On

VI_OFF (0) - Off (Default Value)

voltRang

Variable Type       ViInt16

The command selects the measurement range to be used in voltmeter

mode for the selected channel.

Valid Values:

0  - 400  mV

1  - 4.00 V

2  - 40.0 V

3  - 400  V

4  - 800  V

ohmRang

Variable Type       ViInt16

This command selects the measurement range to be used in ohm-meter

mode.

Valid Values:

0  - 80.0 OHM

1  - 800  OHM

2  - 8.00 K OHM

3  - 80.0 K OHM

4  - 800  K OHM

5  - 8.00 M OHM

6  - 32.0 M OHM

capaRang

Variable Type       ViInt16

This selects the range of measurement to be used in capacitance mode.

Valid Values:

0  - 5.00 nF

1  - 50 nF

2  - 500 nF

3  - 5.00 uF

4  - 50 uF

5  - 500 uF

6  - 5.00 mF

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Input) out of range.

BFFC0003  Parameter 3 (State) out of range.

BFFC0004  Parameter 4 (Coupling) out of range.

BFFC0005  Parameter 5 (Probe) out of range.

BFFC0006  Parameter 6 (Polarity) out of range.

BFFC0007  Parameter 7 (Volts/div) out of range.

BFFC0008  Parameter 8 (Offset) out of range.

BFFC0803  Error scanning response.

BFFC0809  Parameter 9 (Bandwidth) out of range.

BFFC0900  Instrument specific error.

BFFC0907  Parameter 7 (Volts/div) is inconsistent.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_readChToArr

ViStatus mxOX7000_readChToArr (ViSession instrumentHandle,

ViInt16 source,

ViReal64 _VI_FAR waveformArray[],

ViPInt32 number_of_samples,

ViPReal64 xStart, ViPReal64 xIncrement);

Purpose

This function reads the specified channel from the instrument and places

it in an array.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

source

Variable Type       ViInt16

This control selects the source channel.

Valid Values:

0 - CH1  (Default Value)

1 - CH2

2 - CH3

3 - CH4

waveformArray

Variable Type       ViReal64[]

This function specifies the name of array in which to place the

waveform data.

Notes:

(1) The Real Array must be large enough to hold the requested

points.His numberr of elements is 2500.

number_of_samples

Variable Type       ViInt32 (passed by reference)

This function returns the number of valid points read from the

selected channel.

xStart

Variable Type       ViReal64 (passed by reference)

This function returns the time of the first data point in seconds

(with respect to the trigger point).

xIncrement

Variable Type       ViReal64 (passed by reference)

This function returns the incrementing value in seconds for the X

axis.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Source) out of range.

BFFC0003  Parameter 3 (Interpolation) out of range.

BFFC0803  Error scanning response.

BFFC0900  Instrument specific error.

BFFC090C  Channel is not active.

BFFC0910  Invalid file content.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_readChToFile

ViStatus mxOX7000_readChToFile (ViSession instrumentHandle,

ViInt16 source, ViInt16 drives,

ViString fileName,

ViBoolean overwriteFile);

Purpose

This function reads a channel from the instrument and stores it as a

file.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

source

Variable Type       ViInt16

This control selects the source channel.

Valid Values:

0 - CH1  (Default Value)

1 - CH2

2 - CH3

3 - CH4

drives

Variable Type       ViInt16

This control selects the destination drive.

Valid Values:

0 - Computer

1 - MTX3x52 Memory

fileName

Variable Type       ViString

This function specifies the name of the wave data file.

Default Value: ""

Notes:

(1) When storing a file to Computer, precede all backslashes with an

additional backslash, e.g. if the file is in the \cvi\instr

directory, enter the filename as "\\cvi\\instr\\anyfile".

(2) File name must be 15-character long max plus ".TRC" extension.

The extention must be in capital letter.

If not, an error is generated :0xBFFC0004

(3) If the target file already exists and overwrite is not selected,

an error is generated : 0xBFFC090E

overwriteFile

Variable Type       ViBoolean

This function specifies whether or not to write over the specified

file if it already exists.

Valid Range:

VI_OFF (0) - No  (Default Value)

VI_ON  (1) - Yes

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Source) out of range.

BFFC0003  Parameter 3 (Drive) out of range.

BFFC0004  Parameter 4 (Filename) out of specifications.

BFFC0005  Parameter 5 (Overwrite File?) out of range.

BFFC0801  Error writing to file.

BFFC0803  Error scanning response

BFFC0900  Instrument specific error.

BFFC090C  Channel is not active.

BFFC090E  File already exists.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_readCursors

ViStatus mxOX7000_readCursors (ViSession instrumentHandle,

ViInt16 reference, ViPReal64 t1_s,

ViPReal64 t2_s, ViPReal64 dT_s,

ViPReal64 tphase_s, ViPReal64 v1_V,

ViPReal64 v2_V, ViPReal64 dV_V,

ViPReal64 vphase_V);

Purpose

This function reads cursors positions.

All measured quantities are in their basic units.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

reference

Variable Type       ViInt16

This control selects the reference channel for the cursors positions.

Valid Values:

0 - CH1  (Default Value)

1 - CH2

2 - CH3

3 - CH4

t1_s

Variable Type       ViReal64 (passed by reference)

This control outputs the position of the vertical cursor #1.

t2_s

Variable Type       ViReal64 (passed by reference)

This control outputs the position of the vertical cursor #2.

dT_s

Variable Type       ViReal64 (passed by reference)

This control outpus the difference between vertical cursors.

tphase_s

Variable Type       ViReal64 (passed by reference)

This control outpus the difference between vertical cursors.

v1_V

Variable Type       ViReal64 (passed by reference)

This control outpus the position of the horizontal cursor #1.

v2_V

Variable Type       ViReal64 (passed by reference)

This control outpus the position of the horizontal cursor #2.

dV_V

Variable Type       ViReal64 (passed by reference)

This control outpus the difference between horizontal cursors.

vphase_V

Variable Type       ViReal64 (passed by reference)

This control outpus the difference between horizontal cursors.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Reference) out of range.

BFFC0003  Parameter 3 (T1) out of range.

BFFC0004  Parameter 4 (T2) out of range.

BFFC0005  Parameter 5 (dT) out of range.

BFFC0006  Parameter 6 (V1) out of range.

BFFC0007  Parameter 7 (V2) out of range.

BFFC0008  Parameter 8 (dV) out of range.

BFFC0803  Error scanning response

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_readFileToArr

ViStatus mxOX7000_readFileToArr (ViSession instrumentHandle,

ViInt16 drives, ViString fileName,

ViReal64 _VI_FAR waveformArray[],

ViPInt32 number_of_samples,

ViPReal64 xStart,

ViPReal64 xIncrement);

Purpose

This function reads a stored waveform file and places the data in an

array.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

drives

Variable Type       ViInt16

This control selects the source drive.

Valid Values:

0 - Computer

1 - MTX3x52 Memory

fileName

Variable Type       ViString

This function specifies the name of the wave data file.

Default Value: ""

Notes:

(1) When reading a file from Computer, precede all backslashes with

an additional backslash, e.g. if the file is in the \cvi\instr

directory, enter the filename as "\\cvi\\instr\\anyfile".

(2) File name must be 15-character long max + ".TRC" extension.

The extention must be in capital letter.

If not an error is generated :0xBFFC0003

(3) If file does not exist an error is generated : 0xBFFC090D

waveformArray

Variable Type       ViReal64[]

This function specifies the name of array in which to place the

waveform data.

Notes:

(1) The Real Array must be large enough to hold the requested

points.His numberr of elements is 2500.

number_of_samples

Variable Type       ViInt32 (passed by reference)

This function returns the number of valid points read from the

selected file.

xStart

Variable Type       ViReal64 (passed by reference)

This function returns the time of the first data point in seconds

(with respect to the trigger point).

xIncrement

Variable Type       ViReal64 (passed by reference)

This function returns the incrementing value in seconds for the X

axis.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Drive) out of range.

BFFC0003  Parameter 3 (Filename) out of specifications.

BFFC0004  Parameter 4 (Interpolation) out of range.

BFFC0800  Error opening file.

BFFC0803  Error scanning response

BFFC0900  Instrument specific error.

BFFC090D  File not found.

BFFC0910  Invalid file content.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_readInstrData

ViStatus mxOX7000_readInstrData (ViSession instrumentHandle,

ViInt32 numberBytesToRead,

ViChar _VI_FAR readBuffer[],

ViPInt32 numBytesRead);

Purpose

This function reads data from the instrument's output buffer and returns

it to the specified variable in memory.

Notes:

(1) Because the instrument may return both numeric and text data in

response to queries this function returns the data in string format.

(2) If valid data is not available at the instrument's output buffer when

this function is called the instrument will hang up and the function will

not return until it times out.  If the time-out is disabled this function

will hang indefinitely and it may be necessary to reboot the computer to

break out.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

numberBytesToRead

Variable Type       ViInt32

The number of bytes specified by this control should be greater than

or equal to the number of bytes which are to be read from the

instrument. If the actual number of bytes to be read is greater than

the number this control specifies then multiple reads will be

required to empty the instrument's output buffer.

If the instrument's output buffer is not emptied the instrument may

return invalid data when future reads are performed.

Default Value:  50 (See NOTE 2)

Notes:

(1) If NO DATA is available at the instrument's output buffer when

this function is called the instrument will hang up and the function

will not return until it times out.  If the time-out is disabled,

this function will hang indefinitely and it may be necessary to

reboot the computer.

(2) If the number of bytes expected is greater than 50 the value of

this control may be increased. If the actual number of bytes read is

larger than the declared size of the read buffer a run-time error

will be generated.

readBuffer

Variable Type       ViChar[]

The incoming data from the instrument is placed into this variable.

Notes:

(1) Because the instrument may return both numeric and text data in

response to queries this function returns the data in string format.

(2) This function does not overwrite any old data left in the string

variable from the last time the function was called. String data in

LabWindows/CVI is terminated with an ASCII null(0x0) and string

manipulation functions will only recognize data before the ASCII

null.

(3) The declared size of the string variable must be greater than the

actual number of bytes read from the instrument if it is not a

run-time error will be generated.

numBytesRead

Variable Type       ViInt32 (passed by reference)

This variable contains the actual number of bytes read from the

instrument. This is the value which is returned by the read function.

Notes:

(1) If the actual number of bytes read is less than the number of

bytes specified in the Number Bytes To Read control then the output

buffer has probably been emptied. If the read function fails and the

number of bytes read is 0, the most probable cause for the failure is

there was no data available at the instrument's output buffer.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_readMeasurement

ViStatus mxOX7000_readMeasurement (ViSession instrumentHandle,

ViInt16 measurement,

ViInt16 reference, ViPReal64 reading,

ViInt16 reference2_phase);

Purpose

This function forces the instrument to perform selected measurement and

returns the result.

All measured quantities are in their basic units.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

measurement

Variable Type       ViInt16

This control specifies the measurement function.

Valid Values:

0  - Minimum Voltage     [V]

1  - Maximum Voltage     [V]

2  - Peak to Peak        [V] (Default Value)

3  - Low Voltage         [V]

4  - High Voltage        [V]

5  - Amplitude           [V]

6  - Voltage RMS Voltage [V]

7  - Average Voltage     [V]

8  - Rise Overshoot      [%]

9  - Fall Overshoot      [%]

10 - Rise Time           [s]

11 - Fall Time           [s]

12 - Pos Pulse Width     [s]

13 - Neg Pulse Width     [s]

14 - Period              [s]

15 - Frequency           [Hz]

16 - Dutycycle           [%]

17 - N

18 _ Phase

reference

Variable Type       ViInt16

This control selects the reference channel for the specified

measurement.

Valid Values:

0 - CH1  (Default Value)

1 - CH2

2 - CH3

3 - CH4

reading

Variable Type       ViReal64 (passed by reference)

This control outputs the result of the measurement.

All measured quantities are output in their basic units.

reference2_phase

Variable Type       ViInt16

This control selects the reference channel for the specified

measurement.

Valid Values:

0 - CH1  (Default Value)

1 - CH2

2 - CH3

3 - CH4

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Measurement) out of range.

BFFC0003  Parameter 3 (Reference) out of range.

BFFC0004  Parameter 4 (Reading) out of range.

BFFC0803  Error scanning response

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_readMeterMeas

ViStatus mxOX7000_readMeterMeas (ViSession instrumentHandle,

ViInt16 reference, ViPReal64 reading);

Purpose

This function forces the instrument in the meter mode to perform selected

measurement and returns the result.

All measured quantities are in their basic units.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

reference

Variable Type       ViInt16

This control selects the reference channel for the specified

measurement.

Valid Values:

1 - CH1  (Default Value)

2 - CH2

3 - CH3

4 - CH4

reading

Variable Type       ViReal64 (passed by reference)

This control outputs the result of the main measurement for the

selected channel.

All measured quantities are output in their basic units.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Measurement) out of range.

BFFC0003  Parameter 3 (Reference) out of range.

BFFC0004  Parameter 4 (Reading) out of range.

BFFC0803  Error scanning response

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_reset

ViStatus mxOX7000_reset (ViSession instrumentHandle);

Purpose

This function resets the instrument to a known state.

See the Digiscope 2000 Operating Instructions manual for more details.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_revisionQuery

ViStatus mxOX7000_revisionQuery (ViSession instrumentHandle,

ViChar _VI_FAR instrumentDriverRevision[],

ViChar _VI_FAR firmwareRevision[]);

Purpose

This function returns the revision numbers of the instrument driver and

instrument firmware, and tells the user what firmware the driver is

compatible with, for example:

This instrument driver's Revision Number is "Rev 1.0, 04/2005, CVI 6.0"

and is compatible with Instrument Firmware Revision "1.6".

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

instrumentDriverRevision

Variable Type       ViChar[]

This control returns the Instrument Driver Software Revision.

Notes:

(1) The array must contain at least 256 elements ViChar[256].

firmwareRevision

Variable Type       ViChar[]

This control returns the Instrument Firmware Revision.

Notes:

(1) The array must contain at least 256 elements ViChar[256].

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_runAcquisition

ViStatus mxOX7000_runAcquisition (ViSession instrumentHandle,

ViBoolean acquisition);

Purpose

This function controls the acquisition system of the instrument.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

acquisition

Variable Type       ViBoolean

This control valid the acquisition system of the instrument.

Valid Values:

VI_OFF (0) - Stop

VI_ON  (1) - Run   (Default Value)

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Mode) out of range.

BFFC0003  Parameter 3 (Wait?) out of range.

BFFC0803  Error scanning response

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_runAutoset

ViStatus mxOX7000_runAutoset (ViSession instrumentHandle);

Purpose

This function performs an autoset of the instrument.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0803  Error scanning response.

BFFC0900  Instrument specific error.

BFFC090F  No channel is enabled.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_runHardcopy

ViStatus mxOX7000_runHardcopy (ViSession instrumentHandle,

ViBoolean action);

Purpose

This function controls the hardcopy system of the instrument.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

action

Variable Type       ViBoolean

This control selects whether to run the hardcopy or to abort it.

Valid Values:

VI_OFF (0) - Abort

VI_ON  (1) - Run     (Default Value)

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Action) out of range.

BFFC0803  Error scanning response

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_saveSetup

ViStatus mxOX7000_saveSetup (ViSession instrumentHandle, ViBoolean mode,

ViInt16 drives, ViString fileName,

ViBoolean overwriteFile);

Purpose

This function save or recall set-up to or from file.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

mode

Variable Type       ViBoolean

This control selects between Save and Recall mode.

Valid Range:

VI_OFF (0) - Recall

VI_ON  (1) - Save  (Default Value)

drives

Variable Type       ViInt16

This control selects the target/source drive.

Valid Values:

0 - Computer

1 - MTX3x52 Memory  (Default Value)

fileName

Variable Type       ViString

This function specifies the name of the setup data file.

Default Value: ""

Notes:

(1) When reading/writing a file from/to Computer, precede all

backslashes with an additional backslash, e.g. if the file is in the

\cvi\instr directory, enter the filename as "\\cvi\\instr\\anyfile".

(2) File name must be 15-character long max + ".CFG" extension.

The extention must be in capital letter.

If not an error is generated :0xBFFC0004

(3) When saving a setup : if the target file already exists and

overwrite is not selected an error is generated : 0xBFFC090E

(4) When recalling a setup : if file does not exist an error is

generated : 0xBFFC090D

overwriteFile

Variable Type       ViBoolean

This function specifies whether or not to write over the specified

file if it already exists on disk.

Valid Range:

VI_OFF (0) - No  (Default Value)

VI_ON  (1) - Yes

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Mode) out of range.

BFFC0003  Parameter 3 (Drive) out of range.

BFFC0004  Parameter 4 (Filename) out of specifications.

BFFC0005  Parameter 5 (Overwrite File?) out of range.

BFFC0800  Error opening file.

BFFC0801  Error writing to file.

BFFC0803  Error Scanning Response

BFFC0900  Instrument specific error.

BFFC090D  File not found.

BFFC090E  File already exists.

BFFC0910  Invalid file content.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_selfTest

ViStatus mxOX7000_selfTest (ViSession instrumentHandle,

ViPInt16 selfTestResult,

ViChar _VI_FAR selfTestMessage[]);

Purpose

This function runs the instrument's self test routine and returns the

test result(s).

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

selfTestResult

Variable Type       ViInt16 (passed by reference)

This control contains the value returned from the instrument self

test.  Zero means success.  For any other code, see the device's

operator's manual.

selfTestMessage

Variable Type       ViChar[]

This control contains the string returned from the self test. See the

device's operation manual for an explanation of the string's

contents.

Notes:

(1) The array must contain at least 256 elements ViChar[256].

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFF0803  Error scanning response.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_setCountTrigger

ViStatus mxOX7000_setCountTrigger (ViSession instrumentHandle,

ViInt16 auxiliarySource,

ViBoolean edge, ViInt16 coupling,

ViInt16 noiseRejection,

ViReal64 level_volts, ViInt32 events,

ViReal64 holdoff_seconds);

Purpose

This function configures the count trigger parameters of the instrument.

The following settings are available :

- Auxiliary Source

- Edge

- Coupling

- Noise Rejection

- Level

- Events

- Holdoff

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

auxiliarySource

Variable Type       ViInt16

This control selects the counter source.

Valid Values:

1 - CH1  (Default Value)

2 - CH2

3 - CH3

4 - CH4

edge

Variable Type       ViBoolean

This control sets the slope of the specified channel.

Valid Values:

VI_ON  (1) - Pos (Default Value)

VI_OFF (0) - Neg

coupling

Variable Type       ViInt16

This control selects the coupling mode of the specified channel.

Valid Values:

0 - AC (Default Value)

1 - DC

2 - LF Rej

3 - HF Rej

noiseRejection

Variable Type       ViInt16

This control sets the noise rejection of the specified channel.

Valid Values:

0 - OFF  (Default Value)

3 - ON

level_volts

Variable Type       ViReal64

This control sets level of trigger for the specified channel.

Valid Range:

-7.5 * vertSense (vertSense in Volts/div)  to  7.5 * vertSense

(vertSense in Volts/div)

Default Value: 0.0 V

events

Variable Type       ViInt32

This control sets the number of events to count.

Valid Range:

3 to 16 384

Default Value: 3

holdoff_seconds

Variable Type       ViReal64

This control sets the holdoff value.

Valid Range:

40.0 ns to 30s

Default Value: 60.0 ns

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Trigger source) out of range.

BFFC0003  Parameter 3 (Qualifier) out of range.

BFFC0004  Parameter 4 (Counter) out of range.

BFFC0005  Parameter 5 (Events) out of range.

BFFC0905  Parameter 5 (Events) is inconsistent.

BFFC0803  Error scanning response

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_setCursors

ViStatus mxOX7000_setCursors (ViSession instrumentHandle,

ViInt16 cursors, ViInt16 reference,

ViBoolean unattached_cursors,

ViReal64 t1_s, ViReal64 t2_s,

ViReal64 tphase_s);

Purpose

This function sets horizontal cursors.

The following settings are available :

- Cursors

- Reference

- Unattached cursors

- T1

- T2

- TPhase

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

cursors

Variable Type       ViInt16

This control selects the cursors mode.

Valid Values:

0 - OFF  (Default Value)

1 - ON

2 - ON+PHASE

reference

Variable Type       ViInt16

This control selects the reference.

Valid Values:

0 - CH1  (Default Value)

1 - CH2

2 - CH3

3 - CH4

unattached_cursors

Variable Type       ViBoolean

This control selects the unattached cursors.

Valid Values:

VI_OFF (0) - Off

VI_ON  (1) - On   (Default Value)

t1_s

Variable Type       ViReal64

This control sets the position of the vertical cursor #1.

Valid Range:

+/- 100 * horizSense (horizSense in s/div)

t2_s

Variable Type       ViReal64

This control sets the position of the vertical cursor #2.

Valid Range:

+/- 100 * horizSense (horizSense in s/div)

tphase_s

Variable Type       ViReal64

This control sets the position of the vertical cursor #3 (phase).

Valid Range:

+/- 100 * horizSense (horizSense in s/div)

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Clear Cursors) out of range.

BFFC0003  Parameter 3 (Reference) out of range.

BFFC0004  Parameter 4 (T1) out of range.

BFFC0005  Parameter 5 (T2) out of range.

BFFC0006  Parameter 6 (V1) out of range.

BFFC0007  Parameter 7 (V2) out of range.

BFFC0803  Error scanning response

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_setDelayedTrigger

ViStatus mxOX7000_setDelayedTrigger (ViSession instrumentHandle,

ViInt16 auxiliarySource,

ViBoolean edge, ViInt16 coupling,

ViInt16 noiseRejection,

ViReal64 level_volts,

ViReal64 delay,

ViReal64 holdoff_seconds);

Purpose

This function configures the delayed trigger parameters of the

instrument.

The following settings are available :

- Auxiliary Source

- Edge

- Coupling

- Noise Rejection

- Level

- Delay

- Holdoff

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

auxiliarySource

Variable Type       ViInt16

This control selects the auxilary source.

Valid Values:

1 - CH1  (Default Value)

2 - CH2

3 - CH3

4 - CH4

edge

Variable Type       ViBoolean

This control sets the slope of the specified channel.

Valid Values:

VI_ON  (1) - Pos (Default Value)

VI_OFF (0) - Neg

coupling

Variable Type       ViInt16

This control selects the coupling mode of the specified channel.

Valid Values:

0 - AC (Default Value)

1 - DC

2 - LF Rej

3 - HF Rej

noiseRejection

Variable Type       ViInt16

This control sets the noise rejection of the specified channel.

Valid Values:

0 - OFF  (Default Value)

3 - ON

level_volts

Variable Type       ViReal64

This control sets level of trigger for the specified channel.

Valid Range:

-7.5 * vertSense (vertSense in Volts/div)  to  7.5 * vertSense

(vertSense in Volts/div)

Default Value: 0.0 V

delay

Variable Type       ViReal64

This control sets the delay value in seconds.

Valid Range:

40 ns  to  20 s

Default Value: 40 ns

holdoff_seconds

Variable Type       ViReal64

This control sets the holdoff value.

Valid Range:

40.0 ns to 30s

Default Value: 60.0 ns

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Trigger source) out of range.

BFFC0003  Parameter 3 (Qualifier) out of range.

BFFC0004  Parameter 4 (Delay) out of range.

BFFC0803  Error scanning response.

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_setGeneralTrigger

ViStatus mxOX7000_setGeneralTrigger (ViSession instrumentHandle,

ViInt16 triggerSource,

ViBoolean slope, ViInt16 coupling,

ViInt16 noiseRejection,

ViInt16 type, ViReal64 level_volts,

ViReal64 holdoff_seconds);

Purpose

This function configures the general trigger parameters of the

instrument.

The following settings are available :

- Trigger Source

- Slope

- Coupling

- Noise Rejection

- Type

- Level

- Holdoff

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

triggerSource

Variable Type       ViInt16

This control selects the trigger source to configure.

Valid Values:

0 - CH1  (Default Value)

1 - CH2

2 - CH3

3 - CH4

slope

Variable Type       ViBoolean

This control sets the slope of the specified channel.

Valid Values:

VI_ON  (1) - Up (Default Value)

VI_OFF (0) - Down

coupling

Variable Type       ViInt16

This control selects the coupling mode of the specified channel.

Valid Values:

0 - AC (Default Value)

1 - DC

2 - LF Rej

3 - HF Rej

noiseRejection

Variable Type       ViInt16

This control sets the noise rejection of the specified channel.

Valid Values:

0 - OFF  (Default Value)

3 - ON

type

Variable Type       ViInt16

This control selects the type of trigger.

Valid Values:

0 - Triggered

1 - Auto (Default Value)

2 - Single

level_volts

Variable Type       ViReal64

This control sets level of trigger for the specified channel.

Valid Range:

-7.5 * vertSense (vertSense in Volts/div)  to  7.5 * vertSense

(vertSense in Volts/div)

Default Value: 0.0 V

holdoff_seconds

Variable Type       ViReal64

This control sets the holdoff value.

Valid Range:

40.0 ns to 30s

Default Value: 40.0 ns

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Channel) out of range.

BFFC0003  Parameter 3 (Slope) out of range.

BFFC0004  Parameter 4 (Coupling) out of range.

BFFC0005  Parameter 5 (HF Rejection) out of range.

BFFC0006  Parameter 6 (Noise Rejection) out of range.

BFFC0007  Parameter 7 (Type) out of range.

BFFC0008  Parameter 8 (Level) out of range.

BFFC0803  Error scanning response.

BFFC0809  Parameter 9 (Holdoff) out of range.

BFFC0900  Instrument specific error.

BFFC0905  Parameter 5 (HF Rejection) is inconsistent.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_setHardcopy

ViStatus mxOX7000_setHardcopy (ViSession instrumentHandle,

ViInt16 device, ViInt16 port,

ViString filename, ViBoolean type);

Purpose

This function sets hardcopy parameters.

The following settings are available :

- Device

- Port

- Filename

- Color type

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

device

Variable Type       ViInt16

This control selects the device language.

Valid Values:

0  - IBM Proprinter (Default Value)

1  - EPSON ESC/P2

2  - Canon

3  - HP-PCL

4  - Seiko

5  - Postscript

6  - bitmap BMP

7  - GIF

port

Variable Type       ViInt16

This control selects the measurement to display.

Valid Values:

0  - RS232 (Default Value)

1  - Centronics

2  - Network

3  - File

filename

Variable Type       ViString

This control sets the filename where to store the hardcopy.

Default Value: ""

Note : It must be an 8-character name with no extension. Extension is

automatically added according to the device language. See the

Digiscope 2000 Operating Instructions manual for more details.

type

Variable Type       ViBoolean

This control sets the color of pages.

Valid Values:

VI_OFF (0) - Black & White (Default Value)

VI_ON  (1) - Color

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Device) out of range.

BFFC0003  Parameter 3 (Port) out of range.

BFFC0004  Parameter 4 (Filename) out of range.

BFFC0005  Parameter 5 (Type) out of range.

BFFC0006  Parameter 6 (# of pages) out of range.

BFFC0007  Parameter 7 (Page Number) out of range.

BFFC0008  Parameter 8 (# of pens) out of range.

BFFC0803  Error scanning response

BFFC0900  Instrument specific error.

BFFC0906  Parameter 6 (# of pages) is inconsistent.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_setOptionDisplay

ViStatus mxOX7000_setOptionDisplay (ViSession instrumentHandle,

ViInt16 deviceMode,

ViInt16 language);

Purpose

This function configures the display options of the instrument.

The following settings are available :

- Divice Mode

- Language

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

deviceMode

Variable Type       ViInt16

This control selects the principal mode.

Valid Values:

0 - Scope   (Default Value)

1 - Analyser

2 - Recorder

3 - Multimeter

language

Variable Type       ViInt16

This control selects the language used on the instrument.

Valid Values:

0 - English

1 - French  (Default Value)

2 - German

3 - Spanish

4 - Italiano

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Grid) out of range.

BFFC0003  Parameter 3 (Color) out of range.

BFFC0004  Parameter 4 (Screen) out of range.

BFFC0005  Parameter 5 (Menus) out of range.

BFFC0006  Parameter 6 (Language) out of range.

BFFC0803  Error scanning response

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_setPulsTrigger

ViStatus mxOX7000_setPulsTrigger (ViSession instrumentHandle,

ViInt16 triggerPulsate_type,

ViReal64 pulse);

Purpose

This function configures the pulsate trigger mode of the instrument.

The following settings are available :

- Trigger pulsate type

- Pulse

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

triggerPulsate_type

Variable Type       ViInt16

This control selects the trigger pulsate type.

Valid Values:

0 - Equate  (Default Value)

1 - Superior

2 - Inferior

pulse

Variable Type       ViReal64

This control sets the durate  of the pulse.

Valid Values:

20ns   to  20s

Default Value: 20ns

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Source) out of range.

BFFC0003  Parameter 3 (Window) out of range.

BFFC0004  Parameter 4 (Main Level) out of range.

BFFC0005  Parameter 5 (Auxiliary Level) out of range.

BFFC0903  Parameter 3 (Window) is inconsistent.

BFFC0803  Error scanning response.

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_setVideoTrigger

ViStatus mxOX7000_setVideoTrigger (ViSession instrumentHandle,

ViBoolean standard,

ViBoolean polarity, ViInt16 line,

ViReal64 holdoff_seconds);

Purpose

This function configures the video trigger mode of the instrument.

The following settings are available :

- Standard

- Polarity

- Line

- Holdoff

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

standard

Variable Type       ViBoolean

This control selects the standard.

Valid Values:

VI_OFF (0) - 525

VI_ON  (1) - 625  (Default Value)

polarity

Variable Type       ViBoolean

This control selects the polarity.

Valid Values:

VI_OFF (0) - Pos (+)  (Default Value)

VI_ON  (1) - Neg (-)

line

Variable Type       ViInt16

This control selects the number of the line.

Default Value: 1

holdoff_seconds

Variable Type       ViReal64

This control sets the holdoff value.

Valid Range:

40.0 ns to 30s

Default Value: 60.0 ns

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Trigger Source) out of range.

BFFC0003  Parameter 3 (Standard) out of range.

BFFC0004  Parameter 4 (Polarity) out of range.

BFFC0005  Parameter 5 (Field) out of range.

BFFC0006  Parameter 6 (line) out of range.

BFFC0803  Error scanning response

BFFC0900  Instrument specific error.

BFFC0906  Parameter 6 (Line) is inconsistent.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_setWaveformDisplay

ViStatus mxOX7000_setWaveformDisplay (ViSession instrumentHandle,

ViInt16 minMax,

ViInt16 average_rate,

ViInt16 displayMode,

ViBoolean repetitiveSignal);

Purpose

This function configures the waveform display of the instrument.

The following settings are available :

- Min/Max acquisition

- Average rate

- Mode

- Repetitive Signal

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

minMax

Variable Type       ViInt16

This control Valids the Min/Max Acquisition.

Valid Values:

1 - Normal_OFF   (Default Value)

0 - Envelope_ ON

average_rate

Variable Type       ViInt16

This control sets the average rate.

Valid Values:

0 - No averaging (Default Value)

1 - 2

2 - 4

3 - 16

4 - 64

displayMode

Variable Type       ViInt16

This control selects the display mode.

Valid Values:

0 - Vector   (Default Value)

1 - Envelope

repetitiveSignal

Variable Type       ViBoolean

This control selects the repetitive signal.

Valid Values:

VI_OFF (0) - Off

VI_ON  (1) - On   (Default Value)

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Mode) out of range.

BFFC0003  Parameter 3 (Average rate) out of range.

BFFC0004  Parameter 4 (Persistence) out of range.

BFFC0005  Parameter 5 (Interpolation) out of range.

BFFC0803  Error scanning response

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_verticalSetup

ViStatus mxOX7000_verticalSetup (ViSession instrumentHandle,

ViInt16 input, ViBoolean state,

ViInt16 coupling, ViInt16 probe,

ViInt16 voltDiv, ViReal64 offset_volts,

ViBoolean bandwidth);

Purpose

This function configures the selected input channel of the instrument.

The following settings are available :

- Channel

- State

- Coupling

- Probe

- Range

- Offset

- Bandwidth limitation

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

input

Variable Type       ViInt16

This control selects the channel to configure.

Valid Values:

1 - CH1  (Default Value)

2 - CH2

3 - CH3

4 - CH4

state

Variable Type       ViBoolean

This control selects the state of the specified channel.

Valid Values:

VI_ON  (1) - On  (Default Value)

VI_OFF (0) - Off

coupling

Variable Type       ViInt16

This control selects the coupling of the specified channel.

Valid Values:

0 - AC

1 - DC  (Default Value)

2 - GND

probe

Variable Type       ViInt16

This control selects the probe of the specified channel.

Valid Values:

1 - x 1  (Default Value)

10 - x 10

100- x 100

1000 - x 1000

voltDiv

Variable Type       ViInt16

This control sets vertical sense for the specified channel.

Valid Values:

0  - 2.50mV/div

1  - 5.00mV/div

2  - 10.0mV/div

3  - 20.0mV/div

4  - 50.0mV/div

5  - 100mV/div

6  - 200mV/div

7  - 500mV/div

8  - 1V/div     (Default Value)

9  - 2V/div

10 - 5V/div

11 - 10V/div

12 - 20V/div

13 - 50V/div

14 - 100V/div

15 - 200V/div

16 - 500V/div

17 - 1kV/div

Note : This control value must be consistent with the probe value.

See the Digiscope 2000 Operating Instructions manual for more

details. If it is inconsistent an error is generated :

VI_ERROR_INCONSISTENT_PARAMETER7 (0xBFFC0907)

offset_volts

Variable Type       ViReal64

This control sets the signal offset of the specified channel.

Valid Ranges:

vertical sense       offset range

2.5mV/div              +/- 80mV

5mV/div              +/- 80mV

10mV/div             +/- 400mV

20mV/div             +/- 400mV

50mV/div             +/- 2V

100mV/div            +/- 2V

200mV/div            +/- 2.5V

500mV/div            +/- 12.5V

1V/div               +/- 12.5V

2V/div               +/- 64V

5V/div               +/- 64V

10V/div              +/- 64V

20V/div              +/- 640V

50V/div              +/- 640V

100V/div             +/- 640V

200V/div             +/- 6400V

500V/div             +/- 6400V

1kV/div              +/- 6400V

Default Value: 0.0

bandwidth

Variable Type       ViBoolean

This control selects the state of the specified channel.

Valid Values:

VI_ON  (1)   (Default Value)

VI_OFF (0)

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Input) out of range.

BFFC0003  Parameter 3 (State) out of range.

BFFC0004  Parameter 4 (Coupling) out of range.

BFFC0005  Parameter 5 (Probe) out of range.

BFFC0006  Parameter 6 (Polarity) out of range.

BFFC0007  Parameter 7 (Volts/div) out of range.

BFFC0008  Parameter 8 (Offset) out of range.

BFFC0803  Error scanning response.

BFFC0809  Parameter 9 (Bandwidth) out of range.

BFFC0900  Instrument specific error.

BFFC0907  Parameter 7 (Volts/div) is inconsistent.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_writeFileToMem

ViStatus mxOX7000_writeFileToMem (ViSession instrumentHandle,

ViString fileName, ViInt16 target);

Purpose

This function writes a wave data file (in the memory oscilloscope) to the

instrument display memory.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

fileName

Variable Type       ViString

This control selects the target memory.

Default Value: ""

Notes:

(1) File name must be 15-character long max + ".TRC" extension.

The extention must be in capital letter.

If not an error is generated :0xBFFC0003

(2) If file does not exist an error is generated : 0xBFFC090D

target

Variable Type       ViInt16

This control selects the target memory.

Valid Values:

1 - M1  (Default Value)

2 - M2

3 - M3

4 - M4

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Drive) out of range.

BFFC0003  Parameter 3 (Filename) out of specifications.

BFFC0004  Parameter 4 (Overwrite File?) out of range.

BFFC0800  Error opening file.

BFFC0803  Error scanning response

BFFC0900  Instrument specific error.

BFFC090D  File not found.

BFFC0910  Invalid file content.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mxOX7000_writeInstrData

ViStatus mxOX7000_writeInstrData (ViSession instrumentHandle,

ViString writeBuffer);

Purpose

This function writes commands and queries to the instrument to modify

parameters and query device settings.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

writeBuffer

Variable Type       ViString

The user can use this control to send common commands and queries to

the instrument. This control can also be used to write any valid

command to the instrument.

Default Value:  "*CLS\r"

Notes:

(1) The command or query to be sent to the instrument may be a

literal enclosed in double quotes terminated by '\r' i.e. "*CLS\r" or

may be contained in a variable of type string.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.